Your Name: Levi McLean						     Date: September 15th, 2025

| Course: | INFO1251 (Web Server & N/W Fundamentals) Lab # 02 |
| --- | --- |
| Project: |  |
| Due Date: | As per FOL Submission Folders due date/time |

Instructions and how will my lab be marked.

For each question/task provide your written answer within this document. Your written answer or explanation should be 2 to 3 sentences. If you are referencing from other sources, your last sentence will be in your own words, starting with; To summarize, …

You are to use cmd/Command Prompt for this lab assignment.
Help commands … cls – to clear the screen
 /? – after the root command for help/options e.g.:  c:\ping/?
For each new screenshot capture, issue this 1-line command first then the command(s) noted in the lab assignment; c:\ hostname & date /t & time /t

Example screenshot capture:

For each screenshot capture, provide a brief description of the command results.

Your explanations/questions and tasks …
Note, your explanation of each screenshot capture is an important part of your marks.

| Q. |  | Marks |
| --- | --- | --- |
| 1 | Explain the command ‘ping’. Use the ping command with options; -t and -a for this IPv4 address; 8.8.4.4 (2 screenshots) | 3 |
| 2 | Explain the command ‘tracert’. Use the tracert command for this IPv4 address; 8.8.4.4 How many hops did it take to reach 8.8.4.4? (1 screenshot) | 3 |
| 3 | Explain the command ‘netstat’. Use the netstat command with these options; -a, -f, -r (3 screenshots) | 3 |
| 4 | Explain the command ‘route print’. Use the route print command and summarize the output results. (1 screenshot) | 3 |
| 5 | Explain the command ‘nslookup’. Use the nslookup command for google.com and google.ca (2 screenshots) | 3 |
|  | Total | 15 |

Submission Details

Add your written response to the above explanation/tasks and all the screenshot captures with supporting notes below.
Submit this Word document file to the Submission Folders/Dropbox labelled Lab 02.

Question 1

Ping -t 8.8.4.4 will cause your device to “ping” the address 8.8.4.4 until the user stops the command as opposed to the default 4 ping requests before stopping

Ping -a 8.8.4.4 will resolve the hostname associated with the IP address provided. This can be either a domain or a device name, depending on what device you ping, in this case dns.google

Question 2

This command, as the name suggests, traces the route to a designated IP address. It makes “hops” between different devices (or IPs) until it reaches the destination IP or reaches the maximum amount of hops

Question 3

Netstat -a essentially shows all connections with just simple IP addresses and ports in use as well as what state that connection is in

Netstat -f does roughly the same thing as netstat -a except it shows a full domain name for the foreign addresses (the address your device is connecting to)

Netstat -r shows the routing table for the device it is ran on. This means that it shows which interfaces use which ip addresses, subnet masks, gateways and etc.

Question 4

Route print does about the exact same thing as netstat -r by printing the routing table

Question 5

Nslookup is a command used to find information about what device(s) are hosting a certain domain by returning the IP address associated with it.

Google.com and Google.ca are two different domains (their TLDs are different), therefore google.ca will be hosted on a different server with a different IP address than Google.com

©2025 Fanshawe College			Lab #02			INFO1251, 25W